Entrust Private

| Tech Tip ID: MX Series-511 |  |  |  | Release Date: 3/13/2026 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Author: Sebastian White |  |  |  | Revised Date: |  |  |
| Additional Tech Tips and Driver downloads can be found at https://trustedcare.entrust.com/ (A username and password are required to log onto the web page) |  |  |  |  |  |  |
| DATACARD MX Series Systems: |  |  |  |  |  |  |
| MX9100 | MX8100 | MX6100 |  | MX2100 | MX1100 |  |
| X | X | X |  | X | X |  |
| PB8500 | PB6500 |  |  |  | DX5100 | DX3100 |
| Module: UV Cure\ Cardgard |  |  | Controller: All |  |  |  |
| Subject: PFN – Reset switch |  |  |  |  |  |  |

Problem Statement: MFG has identified there are PFN’s in the field that have a different style circuit breaker (809601-015). When in a depressed (open circuit) state, you may have low voltage errors.

Module view		       Depressed (open circuit)	 Pressed (working position)

Solution: Ensure the circuit breaker is in the closed (pressed) position. Please note, when the switch is depress it will appear longer exposing a white band, this indicates that the circuit is open. The item is used in both modules and FRU’s

FRU’s

Cardgard = 575872-001

UV Cure   = 523448-001